# Suggested Design Project Work Sequence

- Show the class how we arrived at the Yachats_Off_Grid_2024.csv data set.  https://nsrdb.nlr.gov/data-viewer 
Gen_Yachats_Off_Grid_2024.ipynb

- Show the class how to fetch data files and other materials in general from https://osu-fifem.github.io/OSU-ESE450/

- Students build plotting tools in Python to explore the load data set.

- Work on a first iteration of the system design.  What do you think is the best way to satisfy the requirements?  Calculate rough sizing and do preliminary (rough) cost analysis.

- Simulation – I will help you when we get to this.  You will write a control law function and a state update function.

  - The control law will determine the power setpoints and generation levels for each controllable device in the system and output the controls as a vector.  e.g. The heating system will be thermostatically controlled, so the control law for the heating system might look something like:
Heater Power=UA*(Tindoor-Toutdoor)

  - The state update function will update the state of any devices and environmental conditions such as house temperature given their previous state and given the control vector. e.g. 
Updated Battery Energy:  Ebatt,n+1=Ebatt,n+Pbatt,charge⋅Δt

- Visualization of simulation results.

- Economic analysis – Using the simulation results, calculate the actual cost of operating the system for the 2024 calendar year.

- What-Ifs.  What if something fails?  How resilient is your system?

- Iterate on design and repeat simulation, visualization, and economic analysis.